DSP Mathematics Reference Notes

Project Vibe-Synth  |  Internal Technical Document  |  v1.0  |  2026-05-10

This document collects the core signal-processing mathematics underpinning the algorithms generated by the Vibe-Synth DSP engine. It is a reference for contributors working on the generation layer, BIBO checker, and Faust template library. Equations use discrete-time notation throughout (n denotes the sample index).

# 1  Signal Basics and Notation

## 1.1  Discrete-time signals

A discrete-time signal x[n] is a sequence of real (or complex) numbers indexed by integer n. In Vibe-Synth, x[n] represents a single audio sample at time t = n / fs, where fs is the sample rate (default 44 100 Hz).

## Unit impulse and unit step

delta[n]  =  1  if n = 0,  else  0

u[n]  =  1  if n >= 0, else  0

Any FIR filter can be fully characterised by its impulse response h[n], the output when x[n] = delta[n].

## 1.2  Convolution

LTI systems produce their output via discrete convolution:

y[n]  =  (x * h)[n]  =  SUM_k  x[k] * h[n - k]

For a length-M FIR filter this sum has exactly M non-zero terms.

# 2  Filter Design

## 2.1  First-order IIR low-pass filter

Difference equation:

y[n]  =  (1 - a) * x[n]  +  a * y[n-1]

Coefficient a for cutoff frequency fc:

a  =  exp( -2*pi * f_c / f_s )

BIBO stable for |a| < 1. In Faust:

lp1(fc) = _ : *(1 - a) : + ~ *(a) with { a = exp(-2*ma.PI*fc/ma.SR); };

## 2.2  Resonant low-pass (biquad)

Second-order section difference equation:

y[n]  =  b0*x[n] + b1*x[n-1] + b2*x[n-2] - a1*y[n-1] - a2*y[n-2]

| Coefficient | Formula |
| --- | --- |
| alpha | sin(w0)/(2*Q) |
| b0 | (1-cos(w0))/2 |
| b1 | 1-cos(w0) |
| b2 | (1-cos(w0))/2 |
| a0 | 1+alpha |
| a1 | -2*cos(w0) |
| a2 | 1-alpha |

Normalise all coefficients by a0. Stability requires |a2| < 1 and |a1| < 1+a2.

# 3  Reverberation Algorithms

## 3.1  Feedback comb filter

y[n]  =  x[n]  +  g * y[n - M]

M = delay length in samples; g = feedback gain. BIBO stability requires |g| < 1.

comb(M, g) = _ : (+ ~ (de.delay(8192, M) * g));

## 3.2  Schroeder reverberator

Combines parallel feedback combs with series all-pass filters. All-pass transfer function:

H(z)  =  (-g + z^(-M)) / (1 - g * z^(-M))

Unity magnitude for all frequencies. Freeverb (ef.reverb_freeverb) adds tuned comb delays and low-pass damping to simulate air absorption.

# 4  Waveshaping and Saturation

## 4.1  Cubic soft clipper

y  =  x - (x^3 / 3)

Taylor expansion of tanh(x) at order 3. Available as ef.cubicnl(drive, offset). Introduces only odd harmonics.

## 4.2  Hard clipping

y = clip(x, -A, A). Introduces odd and even harmonics. In Faust:

hardclip(A) = _ : min(A) : max(-A);

# 5  Amplitude and Frequency Modulation

## 5.1  Tremolo

y[n]  =  x[n] * (1 - d * (1 - (sin(2*pi*fLFO*n/fs) * 0.5 + 0.5)))

## 5.2  Chorus

tau(n)  =  tau_base  +  D * sin(2*pi*fLFO*n/fs)

tau_base is typically 5-30 ms; D is the modulation depth in samples.

# 6  BIBO Stability Conditions

A system is BIBO stable if every bounded input produces a bounded output.

FIR filters are unconditionally BIBO stable.

IIR filters are BIBO stable iff all poles lie strictly inside the unit circle: |zk| < 1.

For y[n] = x[n] + g*y[n-M], the pole is at z = g^(1/M). Stability requires |g| < 1.

BIBOChecker (compiler/bibo_checker.py) runs six heuristic checks: feedback gain >= 1.0, non-productive recursion, missing damping in reverb paths, high filter Q, excessive delay sizes, and large output gains.

# 7  z-Transform Quick Reference

The z-transform converts a discrete-time signal x[n] into X(z):

X(z)  =  SUM_{n=-inf}^{inf}  x[n] * z^(-n)

| Operation / signal | z-Transform |
| --- | --- |
| x[n - k] (delay by k samples) | z^(-k) * X(z) |
| x[n] + y[n] | X(z) + Y(z) |
| a * x[n] | a * X(z) |
| delta[n] (unit impulse) | 1 |
| u[n] (unit step) | z / (z - 1) |
| a^n * u[n] | z / (z - a), ¦z¦ > ¦a¦ |

## 7.1  Transfer function and frequency response

For an LTI system, H(z) = Y(z)/X(z). Frequency response is H(z) evaluated on the unit circle:

H(e^{j*omega})  =  H(z)|_{z = e^{j*omega}}

omega = 2*pi*f/fs is the normalised angular frequency (0 to pi for 0 to fs/2).

# 8  Glossary

BIBO — Bounded-Input Bounded-Output — a stability criterion for LTI systems.

Biquad — A second-order IIR filter section (two poles, two zeros).

Comb filter — A filter whose frequency response resembles the teeth of a comb, produced by adding a delayed copy of the signal.

Difference equation — A recurrence relation expressing the current output sample as a linear combination of past inputs and past outputs.

Faust — A functional programming language for real-time audio DSP that compiles to C++, WebAssembly, and other targets.

FIR — Finite Impulse Response — output depends only on current and past inputs. Always BIBO stable.

IIR — Infinite Impulse Response — output depends on past outputs as well as inputs. Stable iff all poles inside the unit circle.

LFO — Low-Frequency Oscillator — a sine or triangle wave below ~20 Hz used for modulation.

LTI — Linear Time-Invariant — fully characterised by its impulse response.

Pole — A value of z where H(z) goes to infinity. Poles inside the unit circle correspond to stable decaying exponentials.

Q factor — Quality factor — determines the bandwidth of a resonant filter peak. Higher Q = narrower peak.

Waveshaper — A memoryless nonlinear function applied to each sample to introduce harmonic distortion.

2026/05/12 Note：Update this document when new DSP archetypes are added to dsp_templates.json.